- Introduction:

As the environmental preservation considered one of the biggest challenges that occurred in the Egyptian society which represent a potential need to continue life on the Earth planet.

Regarding to the pollution sources and its impact on the environment especially on air quality, the EEAA has gave an additional attention to the air pollution that is so related to the industrial effects, civilization growth ,and other several economical development processes.

Air pollution affects human health passively and cause damage to the surrounding environment. These pollutant particulates could be in solid, liquid, or gaseous form, in addition, they may be natural or caused by human activity.

According to the guidelines of the American Environmental Protection Agency (EPA) and the World Health Organization (WHO), six major pollutants have been put in consideration based on their concentrations and the general pollutants level in the air which determine the ambient air quality.

Ministry of State for Environmental Affairs is concerned with monitoring emissions from major industrial sources as one of the most important steps to control and reduce risks of these pollutants; and in activation of article no. (20) of the Executive Regulation of Environment Law no. 4 / 1994, amended by Law no. 9 /2009, which states that (EEAA shall supervise the establishment and operation of the Environmental Monitoring Networks).

Industrial emission in Egypt focused in a number of hotspot locations, where country industries are located which Represented in cement , petrochemical companies , steel production companies. Petroleum refining companies and manufacturing and other industrial sectors (industry, lead, aluminum, glass…).

The Implementation began of the first phase of this system to monitor emissions from the cement industry all over the Republic and to follow up the emissions using an electronic network for follow-up basis and to ensure the compatibility of the emissions with the allowable limits of the executive regulations of Law No. 4 of 1994 and amended by Law No. 9 of 2009.

- State (S) and impacts (I) related to this theme?

- Comparative Performance tackling the pollution sources across the country:

In view of the essential role of the Ministry of State for Environmental Affairs (MSEA) and its Executive Agency (EEAA) in monitoring air pollutants, identifying their negative impacts and reducing them; the Ministry has established an integrated National Network for monitoring air pollutants composed of 87 stations for monitoring and controlling main air pollutants periodically and continuously since 1998 until now.

Also in according to apply and activate of Article (20) of the Executive Regulation of Environment Law No. 4 / 1994, stipulates that (EEAA shall supervise the establishment and operation of the Environmental Monitoring Networks). So that, National network to monitor industrial companies' emissions has been established. It provides the continuous and effective monitoring of environmental situation along the whole day, achieve control over in the beginning cement companies' stacks and take necessary legal procedures in case of violation which reached to 28 cement company with 126 monitoring site, 5 Fertilizers Company and 1 carbon black company.

- industrial pollution control POLICIES IN egypt

The share of Egyptian industry in GDP is about 40%, and 20% of all jobs are industrial. There are some 30,000 industrial settlements of which 95% in Small and Medium sized enterprises (SMEs). In the last decade, industry has rapidly developed and it is foreseen that industrial output remains growing at a high growth rate of about 7% per year. Industry thus provides higher welfare, more jobs and income to the Egyptian society.

On the other hand, the pollution of the environment in Egypt already reached critical levels around 2000, and little improvement has been achieved so far. Continuous growth of industrial production will lead to even higher environmental stress if no additional policy actions are taken or current policy actions are strengthened to control industrial pollution. In economic terms, it is estimated that the Costs of Environmental Degradation caused by industry are in the range of LE 8 billion per year (2008), whereas industrial remediation costs are estimated at about LE 4 billion.

The Egyptian Pollution Abatement Project (EPAP) is launched with the overall objective to decrease the pollution loads generated by Egyptian industries. An emission reduction of at least 50% is targeted for companies that benefit from EPAP.

This policy statement gives a brief overview the policies of the Egyptian Environmental Affairs Agency (EEAA) towards industrial pollution control (IPC).

- Waste Sector

This sector includes emissions from solid waste disposal sites (SWDS), wastewater handling and waste incineration. Figure (II.10) illustrates the relative shares of different GHGs in the emissions of the waste sector, while figure (II.11) presents amounts and relative shares of different source categories in the waste sector for 2000. The most important gas produced in this sector is CH4. In 2000, CH4 contributed more than 99 % of the total emissions of the sector, with solid waste disposal on land being the most important source category in this sector. In 2000, the contribution of this source category amounted to 11.694 Mt CO2e, represents about 67% of the total GHG emissions from the sector.

Emissions from the waste sector increased from an estimated amount of 6.233 Mt CO2e in 1990 (INC, 1999) to 17.485 Mt CO2e in 2000. This increase is due to a number of factors.

- Solid Waste Disposal on Land

The amount of solid waste generated in 2000 for each governorate was obtained from the Ministry of Local Development, with the waste composition obtained from EEAA publications (Egyptian Environmental Affairs Agency, 2001). The composition of municipal solid waste is primarily organic and food (50-60%) followed by paper (10-25%), plastics (3- 12%), glass (1-5%), metals (1.5-7%) textiles and other material (11-30%). This waste composition was used for estimating the degradable organic carbon in the waste. Municipal solid waste composition and production rates vary significantly with the geographic location in Egypt, as well as with standards of living. According to estimates of the year 2000, municipal solid waste production rates are lower than 0.3 kg/capita/day in low income and rural areas and as high as 1 kg/capita/day in high income urban areas.

The annual growth of municipal solid waste generation was estimated at pproximately 3.4 % in the year 2000 (Egyptian Environmental Affairs Agency, 2001). Data for solid waste amounts generated in the year 2000 were calculated back to obtain amounts for previous years including 1990. The time series of emissions estimates between years 1990 and 2000

- Wastewater Handling

Data on domestic wastewater treatment plants were obtained from the Holding Company for Water and Wastewater and the General Authority for Water and Wastewater. Data for industrial wastewater were very limited and therefore, estimates of the quantities of industrial wastewater provided in the INC were used and extrapolation carried out for values beyond 1990. Default IPCC emission factors were used and assumptions of treated versus untreated wastewater were modified to reflect actual conditions in Egypt. Population statistics of the Central Agency from Public Mobilization and Statistics (CAPMAS), 1999 to 2006 were used for estimating the total organic load.

Tier 1 method from IPCC Good Practice Guidance was used for estimating CH4 emissions from domestic and industrial wastewater. IPCC default value for degradable organic component was used to estimate the total organic load. For industrial wastewater, data on wastewater quantities were only available for food and beverage, textiles and pulp and paper industries. Default IPCC values for CH4 conversion factors and degradable organic component for those industries were used. As for emissions of N2O from human sewage, these were calculated according to the 1996 IPCC guidelines, with the annual per capita protein intake (kg/person/year) for Egypt obtained from FAO official website. The total emissions from wastewater handling for 2000 were equivalent to 5.788 Mt CO2e. The time series of emission estimates for the period 1990 and 2000

Total emissions (1000 ton CO2e/year) from wastewater handling

- INDUSTRIAL SECTOR

The sector of cement in Egypt is responsible for 17 Million tons of CO2 emissions per year, which is about 68% of the total GHG emissions of the Industry Sector. Like cement, fertilizers industry is energy intensive and is responsible of 5 Million tons of CO2e emissions per year, i.e. about 20% of the total emissions of the Industry Sector. Egypt’s Industrial Development Strategy issued in 2006 integrates climate change into national development priorities.

The prime objective of the industrial strategy is to upgrade the Egyptian industrial sector from being technologically excluded to being a technological adopter in the medium term. This will be realized through directly linking the industrial sector to the world technology markets through sustainable technology transfer channels.

The most important and commonly used industrial processes and product use categories in Egypt, particularly with respect to their GHG emissions are cement production, other process uses of carbonates, nitric acid production, and aluminum production. Some of these industrial processes had already partly acquired technologies through the Clean Development Mechanism (CDM) under the Kyoto Protocol.

Over the years, a series of policies & measures have been adopted, so that GHG emissions per unit of product in industrial processes & product use (excluding those related to energy) witnessed a general drop. Examples of these measures are:

- Designating GHGs reducing technology as one of the environmentally sound technologies;

- Creating market for the climate and environment friendly technologies;

- Comprehensive utilization of flue gases, boiler’s steam and every possible heat waste;

- Minimizing ratio between raw materials used and products without drastic changes in specifications;

- Treating tail gases (such as N2O to convert nitrous oxide and NOx in nitric acid into nitrogen, oxygen and water vapor); and

- Controlling the process chain in such a way to reduce the GHG emissions (such as perflurocarbons (PFC’s) emission reduction in aluminum industry).

Selected Examples of Measures to abate GHGs Emissions in the Industrial Sector

| Technical Options | Climate and Other Environmental Effects |
| --- | --- |
| New Technologies and Processes - Hydrogen reduction of metal oxide ores - Carbon-free hydrogen and ammonia production - Non-reactive electrodes for aluminum production - Non-fluorine-based aluminum production | Climate Benefits Savings of CO2 emissions / yr Other Additional Effects Reduction in air pollution from coke |
| Process Improvements - N2O reduction for Nylon Production. - N2O reduction for Urea Production - CF4 reduction in Aluminum Production - HCFC elimination | Climate Benefits - Savings of CO2- equivalent/yr Other Effects - N2O and HCFC reduction will protect ozone layer |
| Material Substitution - Replace metals with plastics - Replace concrete with plastics. - Lighter materials lower transport-related CO2 - Introduce Chemicals made from plant feed stock materials - Cement Blend | Climate Benefits - Has to be determined on case by case basis. Other Effects - Reduction in air pollution |
| Material Recycling/Reuse - Design for disassembly - Design materials for reuse - Material quality cascading | Climate Benefits - Savings of carbon emissions proportional to the increase in Recycling percentage. Other Effects - Less solid waste and lower resource use |

According to the “1996 IPCC “Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories”, the main sources of GHG emissions reported in the industrial processes are; cement industry, limestone and dolomite production and use, iron and steel industry, nitrous oxide (N2O) production for nitric acid and fertilizers production, fluorocarbons (PFCs) from aluminum smelting, emissions related to consumption of halocarbons (HFCs) & other ozone depleting materials, and sulfur hexafluoride (SF6 ) from magnesium production and electrical equipment.

- Industrial GHG Emissions by Gas and Sources

The main emitted Gas in the Egyptian Industry is CO2. The source categories are Cement and Iron &Steel Production sectors. Their emissions are, 17.25 Million tons/year for Cement and 1.576 Million tons for Iron and Steel. The source of N2O emissions is the Nitric Acid production for the Fertilizers industry. Its emissions are 16,266 tons of N2O, equivalent to 5.042 Million tons of CO2e per year. The ammonia not uses in urea production is responsible for 2.74 Million tons of CO2 emissions per year. The Aluminum production emits 160 tons of PFCs, equivalent to 1.077 Million tons of CO2e per year. These main sources contribute 24.945 Million tons of CO2equivalent per year, which is 99.66% of the total emissions of Egypt's industrial sector. Other minor sources are Ozone Depleting Substances and the recorded lime industry in the IDA. They emit 0.051 and 0.031 Million tons of CO2 equivalent per year respectively.

Descending order of active sectors GHG emissions, CO2e, Mton/year

## c) What are the related key drivers (D) and pressures (P) at national level?

- Industrial Pollution

Industry is one of the main sources of pollution I Egypt. About 60% of all CO2 emission originates from industry, and 40% of SO2 emissions. In addition, industry has a negative effect on air quality (in the greater Cairo area), about 10% of suspended dust come from industrial sources. For wastewater and waste, industry also contributes significantly.

An indication of the effects of the fast growth of industrial production on environment is given in the following graph, in which several scenarios for CO2-emissions by industry are shown.

Even with large efforts to increase energy efficiency, annual CO2 emissions of industry will at least increase with 35% between 2006 and 2015. If energy efficiency would not improve at all, emissions almost double. It can be assumed that this upward trend for CO2-emissions also is valid for other industrial pollution, if resource efficiency in industry does not improve drastically.

- Mitigation measures for waste Sector

Six main criteria have been selected for prioritization of mitigation measures in the waste sector. These entail investment costs; payback periods; GHGs emission reductions potentials; duration of implementation; priority in national strategies/programs; and contribution to sustainable development. Mitigation options, concluded from a multi-criteria analysis, were combined for each sub-sector in order to generate a number of scenarios for solid waste and wastewater. The lowest GHG emitting scenario was selected for implementation during the period 2009 to 2025.

Mitigation measures under one or more of appropriate treatment categories, the associated emission reduction potential, and investment costs calculated for 25 years lifetime in simple linear amortization cost, are summarized in tables (III.6) and (III.7) for solid waste and wastewater, respectively (Osama, “Survey on Egypt GHG Emissions, Waste Sector”, 2007).

Summary of identified mitigation measures for solid waste

Summary of identified mitigation measures for wastewater

- Egypt indicators for the Industrial sector

Egypt open market economy attracted industries highly consuming natural resources including NG and associated energy-intensive products like cement and fertilizers industries. In year 2000, the GHG emissions due to cement, iron &steel and fertilizers industries are 95.9% of the total emissions of Egypt industrial sector. This caused the specific emissions indicator GHG emissions per GDP, to rise from 1.85 to 2.6 ton CO2equivalent/Million US$ of industrial production and got the specific GHG emissions per million US$ of the industrial production to increase in year 2000 to 140% of that of 1990.

| *Industry production, at Market prices, Billion LE |  | Industry production, at Market prices, Billion US$ | Industrial processes GHG emissions , Mton CO2e/year | Specific emissions, ton CO2e/MUS$ of GDP | Specific emissions, % of 1990 |
| --- | --- | --- | --- | --- | --- |
| 1990 | 18.02 | 5.63 | 10.10 | 1.79 | 100% |
| 2000 | 63.48 | 17.25 | 27.77 | 1.61 | 90% |

*Source: "Series of basic data of production, investment, employment and wages, 1981-2007"

Promotional Strategies of Policies & Measures for GHG Emissions Reduction.

| Promotion of Integrated Energy Planning (IEP) at the Sectoral Level | - IEP within specific sectors, to help ensure the optimum overall energy mix of sources. - Optimization of indigenous energy resources. - Securing reliable supply of energy at minimum cost to the different sectors of Egypt’s Economy. |
| --- | --- |
| Promotion of Technologies to Diversify Energy Sources & Reduce GHGs Emissions | - Create a market demand for renewables through enhancing efforts to develop cleaner environment friendly energy. - Encourage GHG-reducing technology via promoting renewables R&D. |
| Strengthening Policies and Measures for National Sustainable Energy Development | - Maximizing the use of all available renewable sources of energy. - Switching to using natural gas in substitution to oil for power generation. - Maximizing the share of high-efficient natural gas-fired combined cycle power plants, in addition to introducing large-scale efficient generating units for the traditional thermal power generation. - Minimizing transmission and distribution losses throughout the national electrical networks. - Institutional restructuring of the energy sector. - Enhancing electricity and gas grid-interconnections across borders to neighboring countries. |
| Promotion of Energy Efficiency to Develop Environment-Friendly Energy & Reduce GHG Emissions | - Reinforce energy conservation efforts through integrally managed energy conservation policies and improve efficiency in energy usage. - Fortify energy efficiency policies for the residential and commercial sectors via reinforcing energy efficiency standards, expanding energy efficiency labeling for household appliances, application of energy efficiency code for new buildings and disseminating efficient lighting. - Conserve transportation fuel consumption via promoting cleaner alternative fuel. |
| Strengthening Policies and Measures for GHG Emissions Reduction | - Reinforce GHG-reduction policy via improving methods of farming and animal husbandry in the agriculture and livestock sectors as well as promoting recycling and waste minimization. - Conserve and expand plant sinks through plantation and replantation project. |
| Enhancing Public Participation & Cooperation | - Promote public participation and strengthen partnerships with civil societies and NGOs. - Motivate the public and cooperate with industries to conduct efforts that reduce GHG emissions. |

Total GHGs Emissions according to Various Scenarios (Thousand tons CO2 equivalent)

| Scenario | 2006/2007 | 2016/2017 | 2026/2027 |
| --- | --- | --- | --- |
| Base Case | 168,910 | 339,833 | 681,343 |
| Scenario 2 | 168,910 | 330,221 | 568,371 |
| Scenario 3 | 168,910 | 325,476 | 534,685 |

As a result of the successful implementation of the Support for National Action Plan, SNAP, and the GEF building capacity and GHGs emission reduction projects, the concerns of climate change, within different institutions and Ministries, increased significantly. This could be proved easily by surveying more than twenty projects that emerged during the last decade. It is clearly indicated by the type of undergoing or under preparation projects or studies, all of them are GHGs mitigation actions. These efforts helped to direct the Government of Egypt to consider the continuance of mitigation action projects within different sectors of the national economy.

## d) Which responses (R) have been put in place or are planned at national level for the theme in question?

- Policies and Policy Actions

EEAA is aware of the urgency of strengthening IPC policies and has therefore formulated a set of consistent policies and policy actions. This is shown in the following table:

| Policies | Policy actions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Enforcement of compliance | EIA's and licencing of industrial facilities | Monitoring of industrial emissions | On line monitoring larger industries | Inspection | Compliance Action Plans |
| Cleaner production | Reduction of GHG emissions | Energy conservation | Cleaner fuels | Adopt and implement strategy for CP | Public Private Partnership for CP |
| Economic incentives | Fines | Environmental Protection Fund | Carbon Fund Unit | International grants and loans |  |
| Public disclosure | PROPER | E-govern-ment | Training |  |  |
| Management of industrial hot spots and guidance of industrial development | Planning of Industrial zones | Carrying capacity | Public Private Partnership on a landfill |  |  |
| Continuous review & update of emission standards, monitoring and compliance procedures | New law 9/2009 and its' executive regulations | Extension of the capacity of the inspection |  |  |  |

The legal framework has recently been updated by the Law 9/2009, which replaces the first environmental law 4/1994. Important elements have been added to the new law, next to concentration standards (which cannot control overall pollution loads in a region) load based standards are introduced. Also provisions have been strengthened to manage pollution in industrial areas.

- Strengthening of Policy Actions for Industrial Pollution Control

To encounter the potential growth of industrial pollution the EEAA intends to implement the following policy actions:

- Time limit for the Validity of Permits: In order to create a more dynamic ambiance for EIA, the EEAA will investigate to limit the validity of EIA’s, as this will force industries to submit up-to-date information and may also give an incentive to innovate production processes. It also will be beneficial for overall compliance of industry with environmental legislation;

- Load based emission standards: this enables EEAA to reduce pollution beyond the concentration standards. A basic Cost Benefit analysis shows that such an approach yield large benefits, with a healthy B/C factor;

- Update database on industrial pollution: Good information on the environmental performance of industries is necessary for many policy fields. Therefore, EEAA will strengthen the management of data on industrial pollution, either by revival of the IPIS system or by gradually built up databases on the pollution caused by industries (industrial emission register);

- Online monitoring for all major industries: Online monitoring has proven to be effective in the cement sector. EEAA will gradually enforce online monitoring for all major polluting industries (metal, oil-refining, fertiliser, power, all larger installation exceeding certain pollution levels);

- Improve the functioning of the Environmental Inspection: reform the current practice of the inspection (mainly punitive for a limited number of industrial establishments) to a focus on assisting industry in achieving compliance. Develop the information system as to provide up to date information exchange between regional and central inspection, enabling to define a transparent inspection strategy and planning;

- Re-enforce Compliance Action Plans: this will give enterprises and the authorities opportunities to settle non compliant cases in a reasonable way, avoiding a rigid legislative approach;

- Adopt the strategy for Cleaner Production and make action plans for energy conservation in industry and further shifts from mazut to natural gas: these strategies will enable EEAA to implement CP-projects in co-operation with industry and other stakeholders (gas and energy suppliers). An indicative CBA shows that energy conservation and the shift from mazut to gas leads to large private and societal benefits;

- Further develop the use of economic incentives for IPC in Egypt: As economic arguments often are the basis for non compliant behaviour, further development of economic instruments can help EEAA to take away the economic reasons for non compliance. Either by introducing pollution based taxation or by larger subsidy schemes;

Introduce the PROPER system in Egypt: Compliance beyond the current environmental standards is essential in Egypt given the rapid growth of industry. The public disclosure system PROPER can help with that. EEAA will promote the PROPER system and start with a formal introduction as soon as possible.

- Waste Treatment

The Egyptian relevant ministries, in close collaboration with concerned governorates, have developed several plans and programs over the past ten years to improve the process of dealing with waste reduction, reuse, recycling and/or proper disposal. These plans and programs lead to the reduction in emissions from the waste sector. Yet there are several barriers to achieving the goals of these programs. These comprise the following:

- Although financial support for mitigation of GHGs emissions from the waste sector in Egypt has increased significantly over the last years, it still represents a clear constraint in the implementation of the intended programs.

- The significant dependence on external financial support, as grants and concessionary loans, complicates the planning process, and slows down implementation.

- The limited public awareness about the economic benefits of mitigation options in the waste sector leads to the hesitation of funding institutions to consider waste management activity as an economically viable option.

- Technology transfer represents another barrier mainly in anaerobic digestion technologies, as it needs high capital investment and skills to operate correctly. Some technologies are designed on site-specific bases, which are not optimal for other regions. Highly local skilled experts and extensive studies are needed for proving the suitability and applicability of the technology according to different varying local conditions in Egypt.

- All parties in the waste sector are relatively of limited environmental management experience and the mechanisms for coordination with EEAA are not well established. Furthermore, privatization of the waste sector lacks clear modalities for partnership, particularly with regards to private-public partnership.

Weak enforcement of existing laws and regulations for violations in handling waste reduces the opportunity for achieving the goals of the planned programs.

- PRTR

- Background

A PRTR is an environmental database or inventory of potentially harmful releases or transfer to air, water and soil as well as waste transported off site for treatment or disposal. In addition to collecting data for PRTR from stationary sources.

It was decided to launch the implementation of the Mediterranean PRTR pilot project in the Alexandria Regional Branch Office of the Egyptian Environmental Affairs Agency (EEAA) in view of the experience of the regional branch of EEAA and of the activities already carried out. The project phase I started in December 2001 with a Kick-off meeting, held in Alexandria and continued during 2002 with different activities: training, software development, data acquisition and reporting.

The joint project involved three partners: the Alexandria Regional Branch Office of EEAA, responsible for the implementation of the project and the preparation of the report and the follow-up of activities in Alexandria; the industrial partners, who periodically provide information on releases; UNEP/MAP in close cooperation with ICS-UNIDO, who provide technical support to the implementation process.

- PRTR pilot project 2013

Based on the MoU between UNEP and the Ministry of State for Environmental Affairs, UNEP agrees to co-operate with the Ministry with respect to the project: Implementation of full size PRTR project in the Mediterranean coastal regions of Egypt.

The PRTR Project objectives to which the small scale funding will contribute is:

- To strengthen the reporting capacities of by enabling the countries of the Mediterranean, to establish Pollutant transfer and releases registers as to tool to carry inventories of marine pollution inputs from land bases sources.

- Activities to be carried out by the Ministry with the support of the small-scale funding:

- Establish the national PRTR team based on the ToRs provided by MEDPOL composed of national, regional and industry representatives

- Organize one inception/training/kick off meeting to explain to all actors involved the meaning and objectives of PRTR, the different PRTR available in the world.

- Coordinate all activities to be held at the national level and local level related to PRTR

- Setup the MEDPOL procedures (List of chemicals, frequency of reporting, list of industries, industrial sectors, work flow of information

- Interact with industries and explanation of reporting system

- Collect reports on paper

- Input data in the data base

- Compile the annual reporting: use of the on-line reporting system from industries

- Test of query system

- Test of the GIS interface

- Translate PRTR reports and Guidelines in local language as appropriate

- The expected results of the project are:

- PRTR systems developed and operation as tool to support their compliance with pollutant reporting obligations from the industrial sector;

- Country capacities in reporting pollutant releases to the marine environment increased.

- Expected Outputs:

- PRTR for industrial sources established in  pilot areas; List of chemicals to report the annual pollution load discharged to the sea agreed with the industries

- National structure to deal with PRTR set up in a sustainable manner

- PRTR Progress 2013

- A number of 12 companies across 6 industrial sectors have been identified in Alexandria for participation in PRTR project.

- The identified companies have actively participated and already confirmed their interest and willingness to submit reports for their chemical releases.

- The PRTR unit was set up within EEAA to follow up the activities with the participating companies, also a PRTR team has been appointed in each company.

- During the last period, PRTR team have ensured the contacts with the selected companies, coordination was made to provide assistance for:

- Identifying the list of chemicals in each company and collection of information.

- Explaining the PRTR report.

- Discussions on different methods used to estimate pollution load

- Providing the available National Pollution Inventory (NPI) documents for estimation of pollution load in some industrial sectors (Inorganic chemicals, Iron & Steel, Paper industry).

Currently, the PRTR team is working to estimate pollution loads for pollutants/chemicals generated and released to the environment by the selected companies. Different methods are used to achieve estimation of pollution load as: monitoring results, material balance and emission factors.